INTERFASE RS232 - RELEVOS

PROTOCOLO RS232:  9600 – 8 – N – 1

CABLE:  Plug 1/8 en un extremo y DB9 Hembra en el otro extremo (en el plug 1/8, el “Tip” es Recepción y el “Sleeve” es GND, es decir el “Tip” va conectado con el pin3 y el “Sleeve” con el pin 5 del DB9 Hembra)

Cada uno de los cinco relevos se activan y desactivan enviando dos caracteres ASCII, asi:

1.  MODULOS #1

RELEVO 1 – ON:  &B

RELEVO 1 – OFF:  &A

RELEVO 2 – ON:  &D

RELEVO 2 – OFF:  &C

RELEVO 3 – ON:  &F

RELEVO 3 – OFF:  &E

RELEVO 4 – ON:  &H

RELEVO 4 – OFF:  &G

RELEVO 5 – ON:  &J

RELEVO 5 – OFF:  &I

2.  MODULOS #2

RELEVO 1 – ON:  %B

RELEVO 1 – OFF:  %A

RELEVO 2 – ON:  %D

RELEVO 2 – OFF:  %C

RELEVO 3 – ON:  %F

RELEVO 3 – OFF:  %E

RELEVO 4 – ON:  %H

RELEVO 4 – OFF:  %G

RELEVO 5 – ON:  %J

RELEVO 5 – OFF:  %I

3.  MODULOS #3

RELEVO 1 – ON:  $B

RELEVO 1 – OFF:  $A

RELEVO 2 – ON:  $D

RELEVO 2 – OFF:  $C

RELEVO 3 – ON:  $F

RELEVO 3 – OFF:  $E

RELEVO 4 – ON:  $H

RELEVO 4 – OFF:  $G

RELEVO 5 – ON:  $J

RELEVO 5 – OFF:  $I

4.  MODULOS #4

RELEVO 1 – ON:  #B

RELEVO 1 – OFF:  #A

RELEVO 2 – ON:  #D

RELEVO 2 – OFF:  #C

RELEVO 3 – ON:  #F

RELEVO 3 – OFF:  #E

RELEVO 4 – ON:  #H

RELEVO 4 – OFF:  #G

RELEVO 5 – ON:  #J

RELEVO 5 – OFF:  #I

5.  MODULOS #5

RELEVO 1 – ON:  !B

RELEVO 1 – OFF:  !A

RELEVO 2 – ON:  !D

RELEVO 2 – OFF:  !C

RELEVO 3 – ON:  !F

RELEVO 3 – OFF:  !E

RELEVO 4 – ON:  !H

RELEVO 4 – OFF:  !G

RELEVO 5 – ON:  !J

RELEVO 5 – OFF:  !I

6.  MODULOS #6

RELEVO 1 – ON:  (B

RELEVO 1 – OFF:  (A

RELEVO 2 – ON:  (D

RELEVO 2 – OFF:  (C

RELEVO 3 – ON:  (F

RELEVO 3 – OFF:  (E

RELEVO 4 – ON:  (H

RELEVO 4 – OFF:  (G

RELEVO 5 – ON:  (J

RELEVO 5 – OFF:  (I

7.  MODULOS #7

RELEVO 1 – ON:  )B

RELEVO 1 – OFF:  )A

RELEVO 2 – ON:  )D

RELEVO 2 – OFF:  )C

RELEVO 3 – ON:  )F

RELEVO 3 – OFF:  )E

RELEVO 4 – ON:  )H

RELEVO 4 – OFF:  )G

RELEVO 5 – ON:  )J

RELEVO 5 – OFF:  )I

8.  MODULOS #8

RELEVO 1 – ON:  *B

RELEVO 1 – OFF:  *A

RELEVO 2 – ON:  *D

RELEVO 2 – OFF:  *C

RELEVO 3 – ON:  *F

RELEVO 3 – OFF:  *E

RELEVO 4 – ON:  *H

RELEVO 4 – OFF:  *G

RELEVO 5 – ON:  *J

RELEVO 5 – OFF:  *I

9.  MODULOS #9

RELEVO 1 – ON:  +B

RELEVO 1 – OFF:  +A

RELEVO 2 – ON:  +D

RELEVO 2 – OFF:  +C

RELEVO 3 – ON:  +F

RELEVO 3 – OFF:  +E

RELEVO 4 – ON:  +H

RELEVO 4 – OFF:  +G

RELEVO 5 – ON:  +J

RELEVO 5 – OFF:  +I

10.  MODULOS #10

RELEVO 1 – ON:  /B

RELEVO 1 – OFF:  /A

RELEVO 2 – ON:  /D

RELEVO 2 – OFF:  /C

RELEVO 3 – ON:  /F

RELEVO 3 – OFF:  /E

RELEVO 4 – ON:  /H

RELEVO 4 – OFF:  /G

RELEVO 5 – ON:  /J

RELEVO 5 – OFF:  /I

11.  MODULOS #11

RELEVO 1 – ON:  =B

RELEVO 1 – OFF:  =A

RELEVO 2 – ON:  =D

RELEVO 2 – OFF:  =C

RELEVO 3 – ON:  =F

RELEVO 3 – OFF:  =E

RELEVO 4 – ON:  =H

RELEVO 4 – OFF:  =G

RELEVO 5 – ON:  =J

RELEVO 5 – OFF:  =I

12.  MODULOS #12

RELEVO 1 – ON:  ?B

RELEVO 1 – OFF:  ?A

RELEVO 2 – ON:  ?D

RELEVO 2 – OFF:  ?C

RELEVO 3 – ON:  ?F

RELEVO 3 – OFF:  ?E

RELEVO 4 – ON:  ?H

RELEVO 4 – OFF:  ?G

RELEVO 5 – ON:  ?J

RELEVO 5 – OFF:  ?I